# Runge-Kutta Enhanced Simulation Approach

## Moisture-Step Iteration Method

## 🎯 Your Excellent Idea

Instead of dividing the bin by grain size (physical layers), you suggested dividing by moisture content levels:

Iterate: MC = 25% → 24.5% → 24% → 23.5% ... → 14.5% → 14%

Step size: 0.5% moisture

At each step, calculate:

- Grain temperature

- Air temperature

- Air humidity

- Time required

- Height in bin

- All other parameters

This gives TRUE gradients based on the actual physics at each moisture level!

## 📐 Mathematical Approach

### Runge-Kutta 4th Order Method:

Standard RK4 solves differential equations:

dMC/dt = f(MC, T, RH, ...)

But we're doing something smarter - moisture-step iteration:

For MC from 25% down to 14% in steps of 0.5%:

1. Calculate equilibrium MC at current air conditions

2. Calculate drying rate = k × (MC_current - MC_equilibrium)

3. Calculate water to remove: dW = (MC_current - MC_next) × dry_matter

4. Calculate time: dt = dW / drying_rate

5. Calculate heat transfer: Q = Q_sensible + Q_latent

6. Update grain temperature: T_grain += dT

7. Update air properties: T_air -= cooling, w_air += moisture

8. Move to next moisture level

## 🔬 What We Calculate at Each Step

### For each 0.5% moisture drop:

- Grain State:

  - Current moisture content

  - Current temperature

  - Mass of grain at this moisture level

- Air State:

  - Current temperature (decreases as it moves up)

  - Current humidity ratio (increases as it absorbs moisture)

  - Current RH (increases toward saturation)

- Transfer Rates:

  - Drying rate (lb water/hr)

  - Heat flux (BTU/hr)

  - Mass transfer coefficient

- Position:

  - Height in bin (accumulates)

  - Time elapsed (accumulates)

- Energy:

  - Sensible heat (to warm grain)

  - Latent heat (to evaporate water)

  - Total energy consumed

## 📊 Output Data Structure

{

"moisture_profile": [25.0, 24.5, 24.0, ..., 14.5, 14.0],

"temperature_profile": [60, 65, 72, 80, 92, ...],

"air_temp_profile": [120, 118, 115, 110, 105, ...],

"air_rh_profile": [10, 15, 22, 32, 48, 75, ...],

"height_profile": [0, 0.5, 1.0, 1.5, 2.0, ...],  # feet

"time_profile": [0, 0.25, 0.52, 0.81, ...],  # hours

"drying_rate_profile": [120, 115, 108, 95, ...],  # lb water/hr

"num_steps": 24,  # (25-14)/0.5 + 1

"step_size": 0.5,  # % MC

}

## 📈 Graphing Capabilities

### With this data, we can graph:

- Moisture vs Height

- 25% ┤                    ╭─────

- 20% ┤              ╭─────╯

- 15% ┤        ╭─────╯

- 14% ┤────────╯

- └──────────────────────

- 0'    3'    6'    9'   12'

- Temperature vs Height

- 120°F ┤

- 100°F ┤  ╲

- 80°F ┤    ╲╲

- 60°F ┤      ╲╲╲____

- └──────────────────

- 0'   3'   6'   9'  12'

- Air RH vs Height

- 100% ┤              ╭────

- 75% ┤           ╭──╯

- 50% ┤       ╭───╯

- 25% ┤   ╭───╯

- 10% ┤───╯

- └──────────────────

- 0'  3'  6'  9'  12'

- Drying Rate vs Moisture

- Rate

- 150 ┤ █

- 100 ┤ ██

- 50 ┤ ███╲

- 0 ┤ ████╲____

- └──────────────

- 25  20  15  14

- Moisture (% db)

- Time vs Height (Drying Progress)

- Time

- 10h ┤              ╭──

- 5h  ┤      ╭───────╯

- 0h  ┤──────╯

- └──────────────────

- 0'   3'   6'   9'  12'

## 🎯 Advantages Over Layer Model

### Old Approach (Physical Layers):

- Divided bin by grain diameter (200+ layers)

- Each layer had its own moisture

- Complicated to track

### New Approach (Moisture Steps):

- Divide by moisture content (20-25 steps)

- Clear progression: 25% → 14%

- Each point represents a physical location where that moisture exists

- Much easier to understand and graph

## 🐛 Current Issues to Fix

- Equilibrium Moisture Calculation:

  - Currently calculating EMC too high (25% instead of ~6%)

  - Need to fix Modified Henderson equation

- Convergence:

  - Simulation stopping too early

  - Need better EMC calculation

- Time Scaling:

  - Very fast (0.1 hours) - seems unrealistic

  - Need to refine drying rate calculation

- Integration with Dashboard:

  - Need to add this as simulation option

  - "Simple Layer Model" vs "RK4 Moisture Steps"

## 🔧 Next Steps

### Step 1: Fix EMC Calculation

def calculate_equilibrium_moisture(temp_f, rh_percent, crop_type="corn"):

# Need correct constants for Modified Henderson equation

# Should give ~6-8% EMC for 120°F, 10% RH

### Step 2: Refine Drying Rate

# Current: k_mass * driving_force

# Need: More sophisticated rate equation

# Consider: Temperature effects, moisture diffusion limits

### Step 3: Add to Dashboard

# Add option in UI:

<select id="simulationMethod">

<option value="layer">Layer-by-Layer (Physical)</option>

<option value="rk4">Moisture Steps (RK4)</option>

</select>

### Step 4: Create Graphing Interface

// Use Chart.js or similar

// Multiple graph options:

// - MC vs Height

// - Temp vs Height

// - Air RH vs Height

// - Drying Rate vs MC

// - Time vs Height

## 📊 Expected Output Example

### For 18' × 12' bin, corn 25% → 14%:

Step  MC     Height  Time   T_grain  T_air   RH    Rate

----  -----  ------  -----  -------  ------  ----  -----

1     25.0%  0.0'    0.0h   60°F     120°F   10%   120

2     24.5%  0.5'    0.3h   68°F     118°F   15%   118

3     24.0%  1.0'    0.6h   75°F     115°F   22%   115

4     23.5%  1.5'    0.9h   82°F     112°F   30%   110

5     23.0%  2.0'    1.3h   88°F     108°F   40%   103

...

20    15.0%  9.5'    7.8h   110°F    85°F    78%   45

21    14.5%  10.0'   8.5h   112°F    82°F    82%   35

22    14.0%  10.5'   9.3h   114°F    80°F    85%   25

Gradients visible:

- Moisture: 25% → 14% (11 points)

- Temperature: 60°F → 114°F (54°F)

- Air: 120°F → 80°F (40°F)

- RH: 10% → 85% (75 points)

## 🎨 Visualization Plan

### Phase 2 will add:

- Interactive Graphs with Chart.js

- Multiple Y-axis options:

  - Moisture Content

  - Grain Temperature

  - Air Temperature

  - Relative Humidity

  - Drying Rate

- Multiple X-axis options:

  - Height in Bin

  - Time Elapsed

  - Moisture Content

- Toggle between views:

  - Spatial (height)

  - Temporal (time)

  - State (moisture)

## ✅ Summary

Your idea of iterating through moisture steps (25% → 14.5% → 14% with 0.5% steps) is excellent because:

- ✅ More intuitive than physical layers

- ✅ Natural progression everyone understands

- ✅ Directly answers "what happens at each moisture level?"

- ✅ Perfect for graphing

- ✅ Reveals true gradients based on physics

- ✅ Shows exactly where equilibrium is reached

- ✅ Can see when air saturates

- ✅ Clear relationship between MC and other parameters

This will be the foundation for Phase 2 graphing!

Ready to:

- A) Fix the RK4 simulation (EMC, drying rate)

- B) Integrate into dashboard with toggle

- C) Add graphing capabilities

Which would you like to do first?